SiH_aiml_implementation_

🗺️ Step 1: Get your base data

You’ll typically need:

- DEM (Digital Elevation Model) — for slope (e.g., USGS Earth Explorer)

- Satellite imagery — Sentinel-2 or Landsat-8/9 (for NDVI, NDWI)

- Land use / land cover map — from Bhuvan or ESA CCI

- Mine boundaries / study shapefile — vector layer you draw or download

Download them as .tif (raster) and .shp (vector).

🧭 Step 2: Make all layers use the same coordinate system (CRS)

Think of CRS as how Earth is “flattened” onto a map — every dataset must share the same version.

In QGIS (simple way)

- Open all your files (drag into QGIS).

- Right-click each layer → “Properties” → “Information” → check CRS.

- If they differ (e.g., one is EPSG:4326 and another is EPSG:32643):

  - Right-click → Export → Save Features As…

  - Choose CRS → EPSG:32643 (UTM Zone 43N) (good for J&K)

  - Save the new file — that’s your reprojected version.

Result: All rasters/vectors now align spatially.

📏 Step 3: Make all rasters the same size (resolution)

Example: DEM = 30 m pixels, Sentinel-2 = 10 m.
You must pick one resolution (say 30 m) so they overlay perfectly.

In QGIS:

- Menu → Raster → Projections → Warp (Reproject)

- Set “Target Resolution” = 30 m

- Resampling method:

  - Continuous data (elevation, NDVI) → bilinear

  - Categorical (land use) → nearest neighbor

- Save as new raster.

Now all your rasters have the same pixel size & coordinate grid.

🧮 Step 4: Calculate features (NDVI, NDWI, Slope, etc.)

You’ll now “derive” new layers from the raw data.

(a) Slope from DEM

In QGIS → Raster → Analysis → Slope

- Input DEM

- Output = new raster (values = slope degrees)

(b) NDVI (vegetation)

Formula: (NIR – Red) / (NIR + Red)

Sentinel-2 bands:

- NIR = Band 8

- Red = Band 4

In QGIS:

- Raster Calculator:

- ("B8@1" - "B4@1") / ("B8@1" + "B4@1")

Save as NDVI.tif

(c) NDWI (water presence)

Formula: (Green – NIR) / (Green + NIR)

("B3@1" - "B8@1") / ("B3@1" + "B8@1")

(d) LST (Land Surface Temperature)

This is easiest via Google Earth Engine, because it provides ready-made temperature data (MODIS).

- Log into GEE → Create new script → paste this:

var mod11 = ee.ImageCollection('MODIS/006/MOD11A2')

.select('LST_Day_1km')

.filterDate('2024-01-01','2024-12-31');

var lst = mod11.mean().multiply(0.02).subtract(273.15);

Map.addLayer(lst, {min:20, max:45, palette:['blue','green','red']}, 'LST');

Export.image.toDrive({image: lst, description:'LST', region: your_geometry, scale:1000});

That exports LST.tif to Google Drive.

🧩 Step 5: Align all layers perfectly

In QGIS:

- Raster → Miscellaneous → Align Rasters

- Input all rasters (NDVI, NDWI, Slope, etc.)

- Choose one as “reference”

- Run.

Now all rasters are pixel-by-pixel matched.

📊 Step 6: Convert data into ML-ready table

Each pixel becomes one “row” of data with columns:
NDVI, Slope, NDWI, LST, Distance_to_Mine, Landuse, Label

Option 1 (GUI in QGIS)

- Use Sample Raster Values plugin:

  - Input: your vector of points (e.g., grid or village centers)

  - Layers: NDVI, NDWI, Slope, etc.

  - Output: a .csv with all values.

Option 2 (Python)

import rasterio, geopandas as gpd, pandas as pd

from rasterio.sample import sample_gen

# load rasters

rasters = ["ndvi.tif", "ndwi.tif", "slope.tif"]

datasets = [rasterio.open(r) for r in rasters]

# load sample points (villages or pixels)

points = gpd.read_file("villages.shp")

# extract values

values = [list(sample_gen(ds, points.geometry)) for ds in datasets]

df = pd.DataFrame(values).T

df.columns = ["NDVI","NDWI","Slope"]

⚖️ Step 7: Normalize & encode

You now have numeric columns ready for ML.

from sklearn.preprocessing import StandardScaler, OneHotEncoder

import pandas as pd

# normalize continuous columns

scaler = StandardScaler()

df[['NDVI','Slope','NDWI']] = scaler.fit_transform(df[['NDVI','Slope','NDWI']])

# encode categorical (e.g., landuse)

ohe = OneHotEncoder(sparse_output=False)

encoded = ohe.fit_transform(df[['landuse']])

encoded_df = pd.DataFrame(encoded, columns=ohe.get_feature_names_out())

df = pd.concat([df, encoded_df], axis=1).drop(columns='landuse')

Save final CSV:

df.to_csv("final_dataset.csv", index=False)

🧠 Step 8: Train your ML model (simple start)

Example: Predict deforestation risk using RandomForest.

from sklearn.ensemble import RandomForestClassifier

from sklearn.model_selection import train_test_split

from sklearn.metrics import classification_report

X = df.drop(columns='label')

y = df['label']

X_train, X_test, y_train, y_test = train_test_split(X, y, test_size=0.2)

rf = RandomForestClassifier(n_estimators=200)

rf.fit(X_train, y_train)

print(classification_report(y_test, rf.predict(X_test)))

You can replace label with any target (e.g., contamination hotspot, disease case, etc.).

Step 9: Make maps of predictions

Use QGIS:

- Import your prediction results (CSV with coordinates and predicted class)

- Layer → Add Delimited Text Layer → choose columns for X/Y (longitude/latitude)

- Save as GeoTIFF to visualize hotspots.

🚀 Step 10: You now have a full pipeline

✅ Input data aligned →
✅ Derived features (NDVI, NDWI, slope, LST) →
✅ Normalized & encoded →
✅ ML-ready dataset →
✅ Model trained →
✅ Predictions visualized on a map